Banshee

On Board Coaching, March, 2017

Coaches: Alex Landwehr and Niall Morrow

Mainsail

- Count the puffs in as they arrive, gently release mainsheet to maintain the angle of heel

- Outhaul generally on full

Jib

- Change the system for attaching jib sheets so they move freely around baby stay when tacking

- Check leach cord is not too tight in light conditions

Spinnaker

- Adjust pole height so that the luff of the spinnaker collapses in the middle

- Ease leeward tweaker to release air from spinnaker, particularly on shy runs

- One crew on the brace and one on the sheet, adjust in tandem, foredeck hand can assist trimming by pulling on the brace.

- Use a bridle  to attach topping lift to the pole

Start

- Determine line bias, ducking is better than tacking if having to give way to a boat on starboard

- Sail away from the line for a minute to determine distance to be from the line with a minute to go.

- Better to be too close than too far from the line with a minute to go.

- Use a transit point to assess position relative to the line

General Point

- The momentum of a keelboat will forgive sail-setting errors, unlike a dinghy which stops when settings are wrong.